VIETTEL SEMI

Document number：01

Sync_5M IP Verification Strategy

＜Revision History＞

| Revision | Publication Date | Approval | Collation | Creating |
| --- | --- | --- | --- | --- |
| 1.0 | Oct-25-2026 | - | - | Hungpv79 |

＜Revision content＞

| Revision | No | Revision content |
| --- | --- | --- |
| 0.1 | - | New creation: verification strategy of the Sync_5M IP |

＜Table of contents＞

1.	Introduction	6

1.1.	About this book	6

1.1.1.	Document structure	6

1.1.2.	Attached document	7

1.2.	Input documents	7

1.3.	Output documents	7

1.4.	Abbreviation:	7

1.5.	Notation:	8

1.5.1.	Mark of numeral:	8

1.5.2.	Mark of signal state:	8

1.5.3.	Mark of data size:	8

1.5.4.	Other mark:	8

2.	Verification strategy:	9

2.1.	Outline:	9

2.2.	Policies:	9

2.2.1.	Verification level:	9

2.2.2.	Verification goal:	9

2.2.3.	Verification priority:	10

2.2.4.	Verification methodology/EDA:	11

2.3.	Schedule:	12

2.4.	Test-bench:	13

＜Table of figures＞

Figure 2.2-1: Verification schedule	2

Figure 2.3-1: DUV test-bench	2

＜Table of tables＞

Table 1.1-1: Structure of this document	2

Table 1.1-2: Attached documents list	2

Table 1.2-1: Input documents list	2

Table 1.3-1: List of output documents	2

Table 1.5-1: Mark of numeral	2

Table 1.5-2: Mark of signal state	2

Table 1.5-3: Mark of data size	2

Table 1.5-4: Mark of bus	2

Table 2.1-1: Code coverage target	2

Introduction

About this book

Document structure

For an overview of this document and each chapter, it is shown in Table 1.1-1.

Table 1.1-1: Structure of this document

| Chapter | Title | Overview | Note |
| --- | --- | --- | --- |
| 1 | Introduction | Describes this document: structure, list of input/output documents, abbreviations and notation used in this book. |  |
| 2 | Verification strategy | Describes the verification strategy of the Sync_5M IP: outline, policies (level, goal, priority, methodology/EDA), schedule and testbench. |  |

Attached document

The attached documents of this book, it is shown in Table 1.1-2.

Table 1.1-2: Attached documents list

| No. | Appendix number | Title | Content |
| --- | --- | --- | --- |
| 1 | NA | NA | NA |

Input documents

The input document of this book, it is shown in Table 1.2-1.

Table 1.2-1: Input documents list

| No. | Document name | Issuer | Version | Described name in the book | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Sync_IP_User_Guide.docx | Trungld13 | 0.7 | IP specification | - |
| 2 | Chi_tieu_ky_thuat_Sync_5MHz.docx | Hoangnvn2 | 1.0 | Requirement specification | - |
| 3 | Tai_lieu_kiem_thu_Sync_5MHz.docx | Hoangnvn2 | 1.0 | Reference model specification | - |

Output documents

These are deliverable documents. It is shown in Table 1.3-1.

Table 1.3-1: List of output documents

| No. | Document name | Issuer | File type | Release schedule | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Verification strategy | Hungpv79 | Microsoft word | Oct/E | This book |
| 2 | Verification items | NA | Microsoft excel | NA |  |
| 3 | Verification scenario spec | NA | Microsoft excel | NA |  |
| 4 | Verification result report | NA | Microsoft excel | NA |  |
| 5 | Verification environment instruction document | NA | Microsoft word | NA |  |

Abbreviation:

| No. | Abbreviation | Meaning |
| --- | --- | --- |
| 1 | AWGN | Additive White Gaussian Noise |
| 2 | AXI4 | Advanced eXtensible Interface 4 |
| 3 | CDC | Clock Domain Crossing |
| 4 | CFO | Carrier Frequency Offset |
| 5 | CP | Cyclic Prefix |
| 6 | CORDIC | Coordinate Rotation Digital Computer |
| 7 | DUV | Design Under Verification |
| 8 | EDA | Electronic Design Automation |
| 9 | FFO | Fractional Frequency Offset |
| 10 | IP | Intellectual Property |
| 11 | LTE | Long Term Evolution |
| 12 | NCO | Numerically Controlled Oscillator |
| 13 | NMSE | Normalized Mean Square Error |
| 14 | PSS | Primary Synchronization Signal |
| 15 | RAL | Register Abstraction Layer |
| 16 | RTL | Register Transfer Level |
| 17 | SNR | Signal-to-Noise Ratio |
| 18 | SVA | SystemVerilog Assertion |
| 19 | UVM | Universal Verification Methodology |
| 20 | VIP | Verification IP |
| 21 | W1C | Write-1-to-Clear |

Notation:

- The mark method that will be used in this book is shown below.

Mark of numeral:

- The mark of numeral is shown in Table 1.5-1.

Table 1.5-1: Mark of numeral

| No. | Cardinal number | Description | Example |
| --- | --- | --- | --- |
| 1 | Hexadecimal numbers | In software: 0x is used as a prefix | 0xA |
|  |  | In hardware: 'h/’H is used as a prefix | 'hA |
| 2 | Binary numbers | ‘b/’B is used as a prefix | ‘b1010 |
| 3 | Decimal numbers | Nothing is applied | 10 |

Mark of signal state:

- The mark of signal state is shown in Table 1.5-2.

Table 1.5-2: Mark of signal state

| No | Term | Meaning | Remarks |
| --- | --- | --- | --- |
| 1 | Active | The signal is effective. | Active-low: suffix "n" |
| 2 | Inactive | The signal is not effective. |  |
| 3 | Asserting | Activate the signal. |  |
| 4 | Negating | Make the signal inactive. |  |

Mark of data size:

- The mark of data size is shown in Table 1.5-3.

Table 1.5-3: Mark of data size

| No | Data size | Meaning | Remarks |
| --- | --- | --- | --- |
| 1 | Byte | 8 bits |  |
| 2 | Half word | 16 bits | I or Q |
| 3 | Word | 32 bits | tdata, AXI4 register |
| 4 | Double word | 64 bits |  |

Other mark:

- Mark of bus signal is shown in Table 1.5-4.

Table 1.5-4: Mark of bus

| No | Term | Meaning | Remarks |
| --- | --- | --- | --- |
| 1 | [m:n] | From m to n with increment/decrement of 1 unit Ex: [2:5] represents a 2, 3, 4 and 5 |  |

Verification strategy:

Outline:

- DUV: Sync_5M IP (top-level Synchronization_4G_top), the timing synchronization IP for LTE 5 MHz (7.68 Msps, FFT 512, half-frame of 38400 samples). The IP finds the PSS position (coarse and fine search), tracks it once per half-frame, compensates the fractional frequency offset (FFO) and outputs phase-compensated data with CP (fixed latency of 4087 samples).

- Interfaces: AXI4 slave for 6 registers (clk domain), one level interrupt, AXI4-Stream style sample input and output without tready (clk_250MHz domain), half_frame_trigger, and 9 status pins per sync call.

- Requirement (Requirement specification v1.0): the IP RTL shall be bit-exact with the reference model, i.e. NMSE = −∞ dB for every quantity in every scenario. NMSE ≤ −60 dB is accepted only for vector quantities and only when the rounding difference has been agreed.

- Scope of the reference model: the sync block only (8 outputs of each call and the intermediate quantities in Table 4.2 of the Requirement specification). CFO estimation/compensation is not covered by the reference model. FFO compensation, m_axis data, AXI4 registers, interrupt, CDC and software reset are therefore verified against the IP specification (Sync_IP_User_Guide v0.7).

Policies:

Verification level:

- DUV block verification: Synchronization_4G_top (IP top-level) is the DUV.

Verification goal:

- Scope:

- Black-box verification: all top-level ports are checked against the reference model and the IP specification: s_axis input, m_axis output, status pins (sync_flag, align_flag, sync_shift, sync_hop, sync_state, sync_mode, coarse_position, fine_position, sync_update), pos/pos_valid/tracking_error, AXI4 slave registers and interrupt.

- White-box verification: intermediate quantities listed in Table 4.2 of the Requirement specification (downsampled buffer after scaling, metric M(τ), coarse peak value/position, fine window after scaling, metric M_f(j), fine peak value/position, scaling shift) are checked through internal-signal probes. Probe signal names: NA.

- Configuration/mode:

- Parameters: default configuration (THRESHOLD = 10^7, DATA_WIDTH = 16, POS_WIDTH = 32, FFO_PHA_WIDTH = 18, FFO_N_ITER = 16). Other legal parameter values (e.g. POS_WIDTH = 16): NA.

- Sync block: verify all states (0...3) and modes (INITIAL, STANDARD, TRACKING) and their transitions: immediate attempt, regular call, tracking call, loss of sync, reset and re-acquisition.

- Operation: interrupt mode (SYNC_INTE = 0x3) and polling mode (SYNC_INTE = 0x0); IP enable (SYNC_CTRL.EN) on/off; software reset (SYNC_SWRS).

- Clocks: clk and clk_250_MHz are asynchronous; clk from 25 MHz to 1 GHz; spacing of s_axis_tvalid from 16 to 64 clk_250_MHz cycles, including the real 32/33-cycle pattern of 7.68 Msps.

- Item:

- Code coverage:

- It is shown in Table 2.2-1.

Table 2.2-1: Code coverage target

| No. | Coverage item | Target | Note |
| --- | --- | --- | --- |
| 1 | Toggle | 100% for top input/output ports |  |
| 2 | Line | 100% |  |
| 3 | Cond | 100% |  |
| 4 | Branch | 100% |  |
| 5 | FSM | 100% |  |

- Functional coverage:

- All items described in the verification items document have to be covered (verification items: NA).

- Planned cover points: cyclic delay d (k = 0...4; s = 0, 1, 4351, 4352, 7678, 7679 in each k; d around 27390...27423 and 34530...35100), SNR (no noise, 10, 4, 0, −6 dB), call type (immediate attempt pass/fail, regular call, tracking call, |s| > 30 reset), state/mode transitions, register access type (RW, RO, W1C, WO-P, SLVERR, burst INCR/WRAP), interrupt enable x status combinations, FFO offset up to ±7.2 kHz.

- Timing check:

- Spacing of two s_axis_tvalid samples ≥ 16 clk_250_MHz cycles (4·(MUL_DELAY + 3)).

- pos_valid, tracking_error and sync_update are 1-cycle pulses; m_axis_tvalid is deasserted after tracking_error, software reset or EN = 0.

- STAT.VLD/STAT.ERR and interrupt rise within the specified delay after pos_valid/tracking_error; interrupt = (INTE.ERRE & STAT.ERR) | (INTE.VLDE & STAT.VLD).

- Software reset: core held in reset for CLR_CYCLES + 1 clk_250_MHz cycles, ignored when SYNC_CTRL.EN = 1.

- AXI4 protocol and response codes (SLVERR for invalid or read-only access): VIP protocol checker.

- Connection check:

- Check connection from Synchronization_4G_top to Synchronization_4G_AXI4_wrapper, Synchronization_4G_core, Synchronization_4G_ffo_loop and Synchronization_4G_interrupt by SVA, e.g. the FFO loop between core and ffo_loop (tan_angle to FFO_in, angle to cos_sin), core_rstn generation, gating of s_axis_tvalid and half_frame_trigger by Sync_enable.

- Synchronizer circuit check:

- Check input and output of the synchronizer circuits by SVA: Sync_enable (clk to clk_250_MHz, level), Sync_clear (SYNC_SWRS pulse, toggle synchronizer), pos_valid/tracking_error/pos (clk_250_MHz to clk, toggle synchronizer and pos_hold).

- Minimum spacing: two pos_valid edges > 4 clk cycles, two tracking_error edges > 2 clk cycles, two SYNC_SWRS writes > 2 clk_250_MHz cycles; clock-ratio sweep with several phase offsets.

- Data check:

- Reference model comparison: 8 outputs of every sync call (Table 4.1 of the Requirement specification) and the intermediate quantities (Table 4.2) are compared by NMSE (Table 5.1): −∞ dB required for all scalar quantities; vector quantities −∞ dB (≤ −60 dB only if agreed).

- Register/status check: SYNC_POS equals pos at pos_valid; SYNC_STAT VLD/ERR set and W1C clear (interrupt_clear has priority); reset values and access types of all registers.

- m_axis data check: 38400 contiguous samples per half-frame including CP, first sample = first sample of subframe index 0, fixed delay of 4087 samples. Check of data values after FFO compensation and scaling: expected-data source NA (no reference model for CFO/FFO compensation).

Verification priority:

NA

Verification methodology/EDA:

- Environment:

NA

- Methodology: 
	NA

- EDA tool:

- Tool name: NA

- Tool version: NA

- Number of licenses: NA

Schedule:

NA

Figure 2.3-1: Verification schedule

Test-bench:

NA

Figure 2.4-1: DUV test-bench

Legend of color:

Create new

Model

Reused and Modified