VIETTEL SEMI

Document number：01

Sync_5M IP Verification Strategy

＜Revision History＞

| Revision | Publication Date | Approval | Collation | Creating |
| --- | --- | --- | --- | --- |
| 0.1 | Oct-26-2026 | - | - | Hungpv79 |

＜Revision content＞

| Revision | No | Revision content |
| --- | --- | --- |
| 0.1 | - | New creation: verification strategy of the Sync_5M IP based on Sync_IP_User_Guide v0.7 |

＜Table of contents＞

1.	Introduction	6

1.1.	About this book	6

1.1.1.	Document structure	6

1.1.2.	Attached document	7

1.2.	Input documents	7

1.3.	Output documents	7

1.4.	Abbreviation:	7

1.5.	Notation:	8

1.5.1.	Mark of numeral:	8

1.5.2.	Mark of signal state:	8

1.5.3.	Mark of data size:	8

1.5.4.	Other mark:	8

2.	Verification strategy:	9

2.1.	Outline:	9

2.2.	Policies:	9

2.2.1.	Verification level:	9

2.2.2.	Verification goal:	9

2.2.3.	Verification priority:	10

2.2.4.	Verification methodology/EDA:	11

2.3.	Schedule:	12

2.4.	Test-bench:	13

＜Table of figures＞

Figure 2.2-1: Verification schedule	2

Figure 2.3-1: DUV test-bench	2

＜Table of tables＞

Table 1.1-1: Structure of this document	2

Table 1.1-2: Attached documents list	2

Table 1.2-1: Input documents list	2

Table 1.3-1: List of output documents	2

Table 1.5-1: Mark of numeral	2

Table 1.5-2: Mark of signal state	2

Table 1.5-3: Mark of data size	2

Table 1.5-4: Mark of bus	2

Table 2.1-1: Code coverage target	2

# Introduction

## About this book

### Document structure

For an overview of this document and each chapter, it is shown in Table 1.1-1.

Table 1.1-1: Structure of this document

| Chapter | Title | Overview | Note |
| --- | --- | --- | --- |
| 1 | Introduction | Describes the structure of this book, attached/input/output documents, abbreviations and notation |  |
| 2 | Verification strategy | Describes the outline, policies (level, goal, priority, methodology/EDA), schedule and test-bench for verification of the Sync_5M IP |  |

### Attached document

The attached documents of this book, it is shown in Table 1.1-2.

Table 1.1-2: Attached documents list

| No. | Appendix number | Title | Content |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

## Input documents

The input document of this book, it is shown in Table 1.2-1.

Table 1.2-1: Input documents list

| No. | Document name | Issuer | Version | Described name in the book | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Sync_IP_User_Guide.docx | trungld13 | 0.7 (Draft, 19/10/2026) | IP User Guide | Specification of the DUV (Synchronization_4G_top) |
| 2 | Chi_tieu_ky_thuat_Sync_5MHz.docx | Reference model team | 1.0 (12/10/2026) | Technical specification | Requirement: configuration, input/output and NMSE criteria |
| 3 | Tai_lieu_kiem_thu_Sync_5MHz.docx | Reference model team | 1.0 (12/10/2026) | Test document | Bit-exact algorithm, test flow, scenarios S1-S8, report format |
| 4 | Wideband_LTE-Sync_IP_5MHz.zip | Reference model team | NA | C reference code | Referenced by the User Guide; not received yet |

## Output documents

These are deliverable documents. It is shown in Table 1.3-1.

Table 1.3-1: List of output documents

| No. | Document name | Issuer | File type | Release schedule | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Verification strategy | Hungpv79 | Microsoft word | NA |  |
| 2 | Verification items | NA | Microsoft excel | NA |  |
| 3 | Verification scenario spec | NA | Microsoft excel | NA |  |
| 4 | Verification result report | NA | Microsoft excel | NA |  |
| 5 | Verification environment instruction document | NA | Microsoft word | NA |  |

## Abbreviation:

- IP: Intellectual Property

- DUV: Design Under Verification (Synchronization_4G_top)

- RTL: Register Transfer Level

- PSS / SSS: Primary / Secondary Synchronization Signal

- CP: Cyclic Prefix

- OFDM: Orthogonal Frequency Division Multiplexing

- FFT: Fast Fourier Transform

- HF: Half-frame (5 subframes, 38400 samples at 7.68 Msps)

- CFO / FFO: Carrier / Fractional Frequency Offset

- AWGN: Additive White Gaussian Noise

- SNR: Signal-to-Noise Ratio

- NMSE: Normalized Mean Square Error

- CORDIC: COordinate Rotation DIgital Computer

- NCO: Numerically Controlled Oscillator

- AXI4: Advanced eXtensible Interface 4

- CDC: Clock Domain Crossing

- W1C: Write 1 to Clear

- SVA: SystemVerilog Assertion

- TB: Test-bench

## Notation:

- The mark method that will be used in this book is shown below.

### Mark of numeral:

- The mark of numeral is shown in Table 1.5-1.

Table 1.5-1: Mark of numeral

| No. | Cardinal number | Description | Example |
| --- | --- | --- | --- |
| 1 | Hexadecimal numbers | In software: 0x is used as a prefix | 0xA |
|  |  | In hardware: 'h/’H is used as a prefix | 'hA |
| 2 | Binary numbers | ‘b/’B is used as a prefix | ‘b1010 |
| 3 | Decimal numbers | Nothing is applied | 10 |

### Mark of signal state:

- The mark of signal state is shown in Table 1.5-2.

Table 1.5-2: Mark of signal state

| No | Term | Meaning | Remarks |
| --- | --- | --- | --- |
| 1 | Active | The signal is effective. |  |
| 2 | Inactive | The signal is not effective. |  |
| 3 | Asserting | Activate the signal. |  |
| 4 | Negating | Make the signal inactive. |  |

### Mark of data size:

- The mark of data size is shown in Table 1.5-3.

Table 1.5-3: Mark of data size

| No | Data size | Meaning | Remarks |
| --- | --- | --- | --- |
| 1 | Byte | 8 bits |  |
| 2 | Half word | 16 bits |  |
| 3 | Word | 32 bits |  |
| 4 | Double word | 64 bits |  |

### Other mark:

- Mark of bus signal is shown in Table 1.5-4.

Table 1.5-4: Mark of bus

| No | Term | Meaning | Remarks |
| --- | --- | --- | --- |
| 1 | [m:n] | From m to n with increment/decrement of 1 unit Ex: [2:5] represents a 2, 3, 4 and 5 |  |

# Verification strategy:

## Outline:

- Requirement: the reference model team defines the requirement in the Technical specification v1.0. At every call of the Sync block, the RTL output must match the reference model, evaluated by NMSE. The primary requirement is bit-exact (NMSE = -inf dB).

- DUV: Synchronization_4G_top (Sync_5M IP, User Guide v0.7 Draft). It performs LTE PSS-based timing synchronization on a 7.68 Msps I/Q stream (5 MHz, FFT 512, half-frame 38400 samples): coarse search (4x downsample, 9600 lags), fine search (64 lags), tracking at every subframe index 0, FFO compensation and CP-preserving data output with a fixed latency of 4087 samples.

- Interfaces: AXI4 slave on clk with 6 registers (SYNC_VERS, SYNC_CTRL, SYNC_SWRS, SYNC_INTE, SYNC_STAT, SYNC_POS) and a level interrupt; on clk_250_MHz: s_axis_*, half_frame_trigger, m_axis_*, pos, pos_valid, tracking_error, 8 status pins (sync_flag, align_flag, sync_shift, sync_hop, sync_state, sync_mode, coarse_position, fine_position) and sync_update. The two clocks are asynchronous.

- Reference: the C reference code and the Test document give the golden values of every call. The designer reports that RTL v0.7 matches the C code bit by bit (217 cases, 2252 calls); this plan is an independent verification and does not re-use the designer results.

- DUV status: v0.7 Draft. Not yet tested with real LTE signals or on FPGA; v0.7 is not re-synthesized (per User Guide).

- Out of the NMSE reference scope: CFO estimation/compensation is a separate block in the Technical specification, but the IP integrates it (FFO loop, phase_angle_calculate, scale_data, m_axis). Golden reference for this part: NA.

- Open issue 1: the Technical specification evaluates each call with the state-in injected from the reference model, while in RTL v0.7 the sequencer keeps the state internally (64K circular memory, replays the C receive chain). Whether per-call state injection is possible at the top, or the comparison is made over a continuous stream using the 8 status pins and sync_update: NA (to be confirmed with the designer).

- Open issue 2: intermediate quantities in Table 4.2 of the Technical specification (downsampled buffer, metric M, fine window, peak values...) are not ports of the IP. Observing them needs hierarchical probes of internal signals: NA (signal names to be provided by the designer).

- Open issue 3: the Technical specification lists state 0..3 (2 = transition), while the User Guide v0.7 only produces 0, 1, 3 (CFO estimation of the C code is not modelled). The expected state sequence is to be confirmed with the model team.

- Open issue 4: Wideband_LTE-Sync_IP_5MHz.zip (C reference code) is not received. NA.

## Policies:

### Verification level:

- DUV block verification: Synchronization_4G_top (IP top-level) is the DUV.

- Sub-blocks (sync_corr, sync_metric, sync_udiv, sync_pss_rom, ffo_loop, up_axi4...): verified through the top-level test-bench; stand-alone sub-block verification: NA.

### Verification goal:

#### Scope:

- Black-box verification: drive s_axis_*, half_frame_trigger, AXI4 slave, clocks and resets; check m_axis_*, pos, pos_valid, tracking_error, the 8 status pins, sync_update and interrupt.

- White-box verification: compare internal intermediate quantities of Table 4.2 of the Technical specification with the reference model by hierarchical probes; feasibility and signal list: NA (open issue 2).

#### Configuration/mode:

- Operating modes: sync_mode INITIAL / STANDARD / TRACKING; sync_state 0..3; acquisition with immediate-try call and normal call, tracking, loss of sync and re-acquisition, software reset, EN off/on, polling and interrupt.

- Parameters: default values (THRESHOLD = 10^7, DATA_WIDTH = 16, ANGLE_WIDTH = 32, DOWN_RATE = 4, MAX_COUNTER = 38400, AXI_DATA_WIDTH = 32, FFO_PHA_WIDTH = 18, FFO_N_ITER = 16). POS_WIDTH = 32 (default) and 16 (minimum). THRESHOLD = 3*10^7 only for no-PSS cases (not bit-exact with the C code). Other values are fixed by elaboration checks.

- Clock and input timing: clk and clk_250_MHz with different ratios (clk 25 MHz to 1 GHz, 250 MHz datapath), s_axis_tvalid spacing 32/33 cycles (real) and 16 to 64 cycles. Spacing below 16 cycles is a negative test.

- Channel: cyclic delay and AWGN only (as in Technical specification); frequency offset up to |df| <= 7.2 kHz for the FFO loop.

#### Item:

- Code coverage:

- It is shown in Table 2.2-1. Targets follow the template; candidates for exclusion: unused AXI signals (awlock, awcache, awprot), unused parameters (MEM_ADDR_WIDTH, ACQ_SKIP_FIRST, MUL_DELAY in datapath), elaborate-error branches, MEM_BLACK_BOX.

Table 2.2-1: Code coverage target

| No. | Coverage item | Target | Note |
| --- | --- | --- | --- |
| 1 | Toggle | 100% for top input/output ports | Top-level ports of Synchronization_4G_top |
| 2 | Line | 100% | Exclusion needs review and justification |
| 3 | Cond | 100% | Exclusion needs review and justification |
| 4 | Branch | 100% | Exclusion needs review and justification |
| 5 | FSM | 100% | All FSMs of the DUV |

- Functional coverage:

- All items in the Verification items document (NA, to be created) must be covered. At least: scenarios S1-S8 of the Technical specification (S1 cyclic delay no noise; S2 cyclic delay with AWGN at SNR 10/4/0/-6 dB; S3 noise only; S4 tracking call with shift -40..+40; S5 long run N >= 40 subframes; S6 delay change +-5/+-31/+-40; S7 PSS cut at the end of the first 5 subframes; S8 delay near the half-frame boundary 27390/27391), plus IP-level items: register map and AXI responses (OKAY/SLVERR), interrupt VLD/ERR and polling, software reset and EN, CDC, reset, FFO, m_axis data path, parameter and clock variants.

- Timing check:

- Check by SVA: spacing of s_axis_tvalid >= 16 clk_250_MHz cycles; sync_update, pos_valid, tracking_error are 1-cycle pulses; m_axis latency 4087 samples; SYNC_STAT.VLD set 4-5 clk cycles (100 MHz) after pos_valid; EN effect after 1 clk + 2-3 clk_250_MHz cycles; SWRS ignored while EN = 1 and core held in reset CLR_CYCLES + 1 cycles; minimum spacing of pos_valid (> 4 clk), tracking_error (> 2 clk) and SWRS writes (> 2 clk_250_MHz cycles). Exact list: NA.

- Connection check:

- Check connection from the top to the wrapper, core, interrupt block and ffo_loop (cos_sin, FFO_in, angle, tan_angle) by SVA. Signal list: NA (depends on RTL hierarchy).

- Synchronizer circuit check:

- Check toggle synchronizers (Sync_clear, pos_valid, tracking_error), level synchronizer (Sync_enable) and the pos_hold to pos_s_r bus transfer by SVA: exactly one destination pulse per source event, pos stable when sampled, over the clock ratios listed above.

- Data check:

- Per call of the Sync block, compare the 8 outputs (sync_flag, align_flag, sync_shift, sync_hop, sync_state, sync_mode, coarse_position, fine_position) with the reference model by NMSE. Pass: NMSE = -inf dB for all scenarios (bit-exact). Tolerance level (NMSE <= -60 dB for vector quantities only) is accepted only if the rounding difference is agreed.

- Check pos at every pos_valid, pos_valid cadence (one per half-frame in tracking), tracking_error only on a defined event.

- Check intermediate quantities of Table 4.2 (scaling shift, scaled buffer, metric M, peaks, fine window): NA (open issue 2).

- Check m_axis data: continuous 38400 samples per half-frame with CP, latency 4087 samples, starts at subframe index 0; golden data for FFO compensation and scaling: NA (open issue 3).

- Check register values at reset and after each access (SYNC_VERS, SYNC_CTRL, SYNC_SWRS, SYNC_INTE, SYNC_STAT, SYNC_POS), W1C behaviour and SLVERR responses.

### Verification priority:

- Order of verification:

- Basic items first: reset values, AXI register access, S1 (no noise, boundary cyclic delays), S8, S7 (immediate-try edge cases), S4 (tracking call).

- Combination items later: S2/S3 (noise), S5 (long run), S6 (delay change, loss and re-acquisition), software reset/EN toggle, interrupt/polling, FFO with frequency offset, clock-ratio and tvalid-spacing sweeps, POS_WIDTH = 16.

### Verification methodology/EDA:

#### Environment:

- New creation (SystemVerilog test-bench). DUV RTL is Verilog (.v; language version: NA). Stimulus and golden files are generated offline by the reference model (C code); the designer test-benches are not re-used.

#### Methodology:

- Directed tests with parameter sweeps (data seed, noise seed, cyclic delay), self-checking with SVA, and file-based comparison with the golden output of the reference model scored by NMSE (script language: NA).

#### EDA tool:

- Tool name: NA (tools used by the designer: Verilator 5.050, Vivado 2025.2 xsim, VCS referenced in the source list)

- Tool version: NA

- Number of licenses: NA

## Schedule:

NA

Figure 2.3-1: Verification schedule

## Test-bench:

Block diagram: NA (to be drawn). Planned components: clock/reset generator (clk, clk_250_MHz); stimulus player (I/Q stream from the reference model with cyclic delay and AWGN, s_axis_tvalid spacing control, half_frame_trigger); AXI4 master BFM and register model; output monitors (m_axis, pos*, status pins, sync_update, interrupt); golden file reader and NMSE scoreboard; SVA bind module (timing, connection, CDC); coverage collector; DUV: Synchronization_4G_top.

Figure 2.4-1: DUV test-bench

Legend of color:

Create new

Model

Reused and Modified